Borisova et al. ms 3424 AM-10-030, July 2010 American Mineralogist

Appendix Table 1. LA-ICP-QMS operating conditions *

| Laser ablation system: |  |
| --- | --- |
| Wavelength | 800 nm |
| Repetition rate | 5 Hz |
| Pulse duration | 55 fs |
| Laser optics | 1.75 Silica lens |
| Incident pulse energy used | 1 mJ/pulse |
| Ablation pit diameter | 100 m |
| ICP-QMS: |  |
| Model | Agilent 7500 ce |
| Forward power | 1550 W |
| Gas flows: |  |
| Plasma | 15 L/m |
| Auxiliary | 0.9 L/m |
| Carrier Ar | 1.14 L/m |
| Carrier He | 0.46 L/m |
| Data acquisition parameters: |  |
| Data acquisition protocol | Time Resolved Analysis |
| Scanning mode | Peak hopping, 1 point per peak |
| Isotopes determined | 7Li, 9Be, 11B 23Na, 27Al, 29Si, 39K, 43Ca, 44Ca, 66Zn, 68Zn, 71Ga, 75As, 85Rb, 88Sr, 89Y, 90Zr, 93Nb, 95Mo, 114Cd, 118Sn, 121Sb, 133Cs, 138Ba, 139La, 140Ce, 144Nd, 172Yb, 181Ta, 208Pb, 232Th,238U |
| Dwell time per isotope | 15 ms |
| Numbers of replicate | 1 |
| Blank analysis | 10 s |
| Analysis time | 120 s |

* See Borisova et al. (2008) for more details.